DEPARTMENT OF CORRECTIONS

MONTANA WOMEN’S PRISON

OPERATIONAL PROCEDURE

| Procedure No.: MWP 2.1.1b | Subject: CONTRACTOR ENTRANCE AND WORK PROCEDURES |  |
| --- | --- | --- |
| Reference: DOC Policy 2.1.1 |  | Page 1 of 3 & 1 attachment |
| Effective Date: 1 September 2006 |  | Revision Date: 02-09-2012, 05-21-2013 |
| Signature: //s// Deputy Warden Robert Paul |  |  |

I.         PURPOSE

To maintain a safe and secure facility, all contractors, services providers, bidders, sub-contractors, and subordinates shall follow this procedure:

II.       DEFINITIONS

Class 1 “Restricted” Tools means tools that may reasonably be considered for their potential to cause serious bodily harm, to affect an immediate escape or to compromise security.  (Ladder, rope, or extension cords in excess of 6 feet in length, hammers, screwdrivers with shanks 6 inches or more in length, wire cutters, bolt cutters, chisels, hacksaw blades, etc.) RamJet hammers and the cartridges will be stored separately with cartridges maintained outside the institution when not in use.

Class 2 “Hazardous” Tools means tools that without alteration may reasonably be considered for their potential to cause serious bodily harm. (Pipe wrenches, metal measuring tapes, 24x16 squares, hand saws, etc.)

Security Manager- The Security Manager is the Deputy Warden in charge of Security and Operations as designated by the Warden to manage the facility security program.

Welding/Hot Works Procedures is any activity which results in sparks, fire, molten slag, or hot material which has the potential to cause fires or explosions.

Examples of Hot Works are Cutting, Brazing, Soldering, Thawing Pipes, Torch Applied Roofing, Grinding and Welding.

Special Hazard Occupancies is any area containing Flammable Liquids, Dust Accumulation, Gases, Plastics, Rubber and Paper Products.

III.      PROCEDURES

A. Contract Worker must: Read this information before doing any work on the outside or within the secure perimeter of this facility.

- Agree to be accountable and liable for all material, tools, products, equipment, etc, brought into the facility. The Contractor representative will provide an inventory of all tools that will be needed for the project.

- The use of Class 1 and Class 2 tools is subject to direct supervision.

- Adher to all procedures and guidelines regarding Hot Works Activities. (see attachment A)

B. 	Security Manager /designee will inspect and inventory all tools introduced to MWP by contractors or others, and will re-inspect and reconcile the inventory list prior to the contractor or other exiting MWP.

C.	Correctional Officer		A correctional officer or other assigned employee will escort contracted and other non-employee workers while working inside the secure perimeter at MWP.

D.	Security Manager/designee	Will dictate the supervision and use of the tools in areas outside the secure perimeter.  These procedures will ensure that all tools and equipment are accounted for, inventoried, and securely stored at the end of the work shift, or before offenders are returned to the work area.  This will ensure that no tools are left behind, work activities are confined to the authorized area, and work is performed with proper security and safety precautions.

At departure, the inventory will be checked to ensure that all tools are present, the entrance log will be marked, and the inventory sheet filed and retained for 30 days.

E.	Contractor will ensure that all contractor employees are able bodied and can work within the secure environment.

Submit a list of names, including dates of birth and social security numbers, of contract and subcontracted employees for background checks. Any employee with a felony conviction will need prior approval from the Director of the Department of Corrections before access to the facility is granted. Any employee with and extensive criminal history, or narcotics related incidents within the past three years will require approval from the Security Major.

F.	Contract Worker appropriate physical distance and non-communication between contracted employees and inmates will be maintained at all times.

1.	This is a TOBACCO FREE facility.  No tobacco or tobacco products will be allowed at the facility.  All tobacco or tobacco products will be stored outside facility grounds and locked in vehicles.

2.	Any employee who enters this facility will not have consumed or be under the influence of alcohol and/or narcotics.

3.	Access is allowed in and out of the locked areas by facility staff only.  No facility keys will be issued.

4.	Universal Precautions are in effect. A situation where blood or bodily fluids are encountered (on floors, walls, clothing, equipment, etc.) shall be handled in accordance with DOC 1.3.35 Bloodborne Pathogens Exposure Control Plan & DOC 4.5.11 Infection Control Program.

G.	Contract Worker Understand inmates have a right to privacy, confidentiality, and to be free from abuse, whether verbal or physical.

- Work in a safe manner through the observance of applicable safety rules, safe work practices, and common sense.

- Recognize that most material, products, equipment, and other items used are to be considered contraband.  All items to be used on the project must be secured at the end of the working day or when changing shifts.  The site shall be kept clean and orderly throughout the duration of the project.

- Uphold federal and state constitutional laws and regulations.

- Agree to immediately correct all situations to the satisfaction of Facility Safety and Security     Personnel.

IV.       CLOSING

Questions concerning this procedure should be directed Security Manager/designee.

V.         ATTACHMENTS

Hot Works Rules and Procedures	                                                            Attachment A

Tools Check List Form for Contractors 		Attachment B

WELDING & CUTTING SAFETY (HOT WORK)

ATTACHMENT A

The Montana Women’s Prison is dedicated to the protection of our employees from occupational injuries and illnesses.  The Montana Women’s Prison is responsible for providing a safe working environment, and the employees have and must assume the responsibility of working safely.

The objective of this program is to supplement our safety policy by providing specific standards regarding Hot Work and to ensure that each employee is adequately trained and fully aware of safety procedures associated with Hot Work.

Welding and Hot Work, such as brazing or grinding presents a significant opportunity for fire and injury.  Company employees or contractors must apply all precautions of this program prior to commencing any welding or hot work.  Reference: OSHA 29 CFR 1910.252

Elimination of injuries and illnesses improves employee morale, improves customer service, improves product quality, and reduces Workers’ Compensation costs.  This policy serves as a tool to increase employee protection, and to reduce jobsite hazards.

Employees are required to comply with the guidelines set forth, and to comply with the instruction of Maintenance Supervisor/Safety Officer.  In the event an unsafe condition arises in the absence of Maintenance Supervisor/Safety Officer, employees should alert the lead person on the jobsite immediately.  Employees should also alert co-workers of any unsafe conditions that arise.

Contractors who may be engaged in Hot Works activities entering the facility will familiarize themselves with this attachment.  For jobs requiring Hot Works, the Maintenance Supervisor/Safety Officer will provide this information along with contractor code of conduct.

Hot Work Procedures   (Attachment A)

- Where practicable all combustibles will be relocated at least 35 feet from the work site.

- Where relocation is impractical, combustibles must be protected with flameproof covers, shielded with metal, guards, curtains, or wet down to help prevent ignition of material.

- Ducts, conveyor systems, and augers that might carry sparks to distant combustibles must be protected or shut down.

- Where cutting or welding is done near walls, partitions, ceilings, or a roof of combustible construction, fire-resistant shields or guards will be provided to prevent ignition.

- If welding is to be done on a metal wall, partition, ceiling, or roof, precautions must be taken to prevent ignition of combustibles on the other side, due to conduction or radiation of heat.

- Where combustibles cannot be relocated on the opposite side of the work, a fire watch person will be provided on the opposite side of the work.

- Welding will not be attempted on a metal partition, wall, ceiling or roof having a covering nor on walls having combustible sandwich panel construction.

- Cutting or welding on pipes or other metal in contact with combustible walls, partitions, ceilings, or roofs will not be undertaken if the work is close enough to cause ignition by combustion.

- In areas where there is dust accumulation of greater than 1/16 inch within 35 feet of the area where welding/hot works will be conducted, all dust accumulation will be cleaned up following the housekeeping program of the facility before welding/hot works are permitted.

- Suitable fire extinguishers must be provided and maintained ready for instant use.

- A fire watch person will be provided during and for two hours past the completion of the welding project.

- A cutting/welding permit will be issued on all welding or cutting outside of designated welding areas.

Cutting or welding will not be permitted in the following situations:

- In areas not authorized by management.

- In sprinklered buildings while such protection is impaired.

- In the presence of potentially explosive atmospheres.

- In areas near the storage of large quantities of exposed, readily ignitable materials.

Welding & Hot Work Fire Prevention Measures

A designated welding area should be established to meet the following requirements:

- Floors swept and clean of combustibles within 35 feet of work area.

- Flammable and combustible liquids and material will be kept 35 feet from work area.

- Adequate ventilation providing 20 air changes per hour, such as a suction hood system should be provided to the work area.

- At least one 10-lb. dry chemical fire extinguisher should be within access of the 35 feet of work area.

- Protective dividers such as welding curtains or non-combustible walls will be provided to contain sparks and slag to the combustible free area.

Requirements For Welding Outside Designated Areas

- Portable welding curtains or shields must be used to protect other workers in the welding area.

- A hot works permit must be completed and complied with prior to welding operation.

- Respiratory protection is mandatory unless an adequate monitored airflow away from the welder and others present can be established and maintained.

- Plastic materials must be covered with welding tarps during welding procedures

- Fire Watch must be provided for all hot work operations.

Welding Standard Operating Procedures (SOP)

The following lists Welding Standard Operating Procedures (SOP) and are applicable for all electric and gas welding.  These SOP are to be posted at each Designated Welding & Hot Work Area for quick reference and review.

SOP—Electric Welding

- Perform Safety Check on all equipment

- Ensure fire extinguisher is charged and available

- Ensure electrical cord, electrode holder and cables are free from defects (no cable splices are allowed within 10 feet of the electrode holder).

- Ensure PPE (welding hood, gloves, rubber boots/soled shoes, and aprons) are available and have no defects.

- Ensure the welding unit is properly grounded.

- All defective equipment must be repaired or replaced before use.

- Remove flammables and combustibles

- No welding is permitted on or near containers of flammable material, combustible material or unprotected flammable structures.

- Place welding screen or suitable barricade around work area to provide a fire safety zone and prevent injuries to passersby (do not block emergency exits or restrict ventilation).

- Ensure adequate ventilation and lighting

- Execute Hot Work Permit procedures

- Set Voltage Regulator no higher than the following for:

- Manual Alternating Current Welders - 80 volts

- Automatic Alternating Current Welders - 100 volts

- Manual or automatic Direct Current Welders -100 volts

- Uncoil and spread out welding cable

- To avoid overheating, ensure proper contact of work leads and connections, remove any metal fragments from magnetic work clamps (to avoid electric shock do not wrap welding cables around a body part and avoid welding in wet conditions)

- Fire watch for one hour after welding & until all welds have cooled

- Perform final fire watch and terminate permit.

SOP—Gas Welding

- Perform Safety Check on all equipment.

- Ensure tanks have gas and fittings are tight.

- Ensure fire extinguisher is charged and available.

- Inspect hoses for defects.

- Ensure PPE (welding hood, gloves, rubber boots/soled shoes, and aprons) are available and have no defects.

- All defective equipment must be repaired or replaced before use.

- Remove flammables and combustibles

- No welding is permitted on or near containers of flammable material, combustible material or unprotected flammable structures.

- Place welding screen or suitable barricade around work area to provide a fire safety zone and prevent injuries to passersby (do not block emergency exits or restrict ventilation).

- Use an authorized Air Filtering Respirator, if required.

- Ensure adequate ventilation and lighting.

- Execute Hot Work Permit procedures.

- Open valves on oxygen and gas tanks to desired flow.

- Shut tank valves & relieve hose pressure.  Store hoses.

- Fire watch for one hour after welding and until all welds have cooled.

- Perform final fire watch and terminate permit.

### Compressed Gas

### Care, Transporting, Moving and Storage

- Valve caps on cylinders must be in place and secured.  Valve caps must not be used for lifting.  Do not pry cylinder caps while frozen.  Loosen caps with warm water.

- Cylinders must be transported on a secured cradle only, and by tilting or rolling them.

- Cylinders must be moved by tilting and rolling them on their bottom edges.  Avoid dropping cylinders or striking other cylinders.

- Cylinders transported by powered vehicles must be secured in a vertical position.

- Regulators must be removed and caps put in place prior to moving cylinders, unless cylinders are secured on a special carrier.

- Proper steadying devices must be used to keep cylinders from falling over while in use.

- Cylinder valves must be closed when cylinders are empty or when cylinders are moved.

- Oxygen cylinders must be stored separated from fuel gas cylinders or combustible materials a minimum distance of 20 feet or by a five foot high non-combustible barrier with a fire-resistance rating of one-half hour.

- Cylinders stored inside buildings must be stored 20 feet from combustible materials where they are well protected, well ventilated, and dry.  Cylinders must not be stored near elevators, stairs or gangways.  Assigned storage locations must prevent cylinders from being knocked over or damaged.

- Cylinders must be kept away from welding or cutting operations to prevent sparks, hot slag, or flame from reaching them.  Fire resistant shields must be used when this is impractical.

- Cylinders must be placed away from electrical circuits.  Do not strike electrodes against a cylinder to strike an arc.

- Cylinders containing oxygen, acetylene or other fuel gas must not be used in confined spaces.

- Cylinders must not be used as rollers or supports.

- Only the gas supplier is authorized to mix gases in a cylinder.  Only Maintenance Supervisor/Safety Officer is authorized to have cylinders refilled.

- No damaged or defective cylinder may be used.

### Use of Fuel Gas

- The Montana Women’s Prison employees will be instructed in the safe use of fuel gas.

- Valves must be opened slightly and closed immediately before a regulator is connected to the cylinder.  This is called “cracking” which clears the valve of dust and dirt.  The

employee must stand to the side of the outlet, not in front.  Valves must be cracked away from welding work, sparks, flames or other sources of ignition.

- Valves must be opened slowly to prevent damage to the regulator.  Valves must not be opened more than 1½ turns.  If a wrench is required it must stay in position in case of emergency for a quick shut off.  Manifold or coupled cylinders must have a wrench available for immediate use.  Do not place objects on top of cylinders, or damage may occur to the safety device or interfere with the quick closing of the valve.

- Cylinders must be closed and the gas released from the regulator before removing the regulator.

- If cylinders, valves, regulators, plugs, or other safety devices are damaged, they must be tagged out of service and removed from the work area.

# Manifolds

- Fuel gas and oxygen manifolds must bear the name of the substance they contain.

- Fuel gas and oxygen manifolds must not be placed in confined spaces, but will be placed in safe, well ventilated, and accessible locations.

- Hose connections must be made so that they cannot be interchanged between fuel gas and oxygen manifolds and supply header connections.  Keep hose connections free of grease and oil, and do not use adapters to interchange hoses.

- Manifold and header hose connections must be capped when not in use.

- Nothing may be placed on manifolds that will damage the manifold or interfere with the quick closing of the valves.

### Hoses

- Fuel gas hose and oxygen hose must be distinguishable from each other, and not be interchangeable.  Contrast may be made by different colors or by surface characteristics distinguishable by the sense of touch.

- Employees will inspect all hoses in use at the beginning of each work shift.  Defective hoses will be tagged and removed from service.

- Hoses subjected to flashback, or which show severe wear or damage must be tested at twice the normal pressure for the hose, but not less than 300 p.s.i.  If defective, the hose must not be used.

- Hose couplings must be disconnected by rotary motion only.

- Hoses stored in boxes must be well ventilated.

- Hoses, cables, and other equipment must be kept clear of passageways, ladders and stairs.

##### Torches

- Torches must be inspected at the beginning of each working shift for leaking shutoff valves, hose couplings, and tip connections.  Defective torches may not be used.

- Clogged torch tip openings must be cleaned.

- Torches may be lit by friction lighters or other approved devices only.

Regulators and gauges

- Pressure regulators and related gauges, must work properly while in use.

Oil and grease hazards

- Oxygen cylinders and fittings must be kept away from oil or grease.  Cylinders and fittings must be kept free from oil or greasy substances and may not be handled with oily hands or gloves.

- Oxygen must not be directed at oily surfaces, greasy clothes, or within a fuel oil or other storage tank or vessel.

## Fire Protection

- Objects to be welded, cut or heated must be moved to a designated safe location.  If the object cannot be easily moved, all moveable fire hazards will be moved or protected.

- If the object to be welded, cut, or heated cannot be moved and if all the fire hazards cannot be removed, positive means must be taken to confine the heat, sparks, and slag, and to protect the immovable fire hazards from them.

- Welding, cutting, or heating must not be performed in the presence of flammable paints, flammable compounds or heavy dust concentrations.

- Fire extinguishers must be immediately available in the work area, free of obstruction, and maintained for instant use.

- When normal fire prevention precautions are not sufficient for the welding, cutting, or heating operation The Maintenance Supervisor/Safety Officer will assign a fire watch.  Sufficient amount of time must be allowed after completion of work to ensure that the possibility of fire does not exist.  The designated fire watch must be trained in fire fighting equipment.

- Gas supplies must be shut off during lunch breaks, overnight, or during shift breaks.  Hoses and torches must be removed from confined spaces.

Training

The Montana Women’s Prison employees involved in Hot Works Activities are required to become familiar with this policy and understand the guidelines regarding Welding and Cutting operations.

Training should include:

- Review of requirements listed in OSHA 1910.252

- Use of Hot Works Permit System

- Supervisor responsibilities

- Fire Watch responsibilities

- Operator responsibilities

- Contractors responsibilities

- Documentation requirements

- Respirator usage requirements

- Fire Extinguisher training

## Conclusion

All employees of Montana Women’s Prison are required to comply with the rules set forth in this written program.  This program is intended to provide the maximum protection for employees of Montana Women’s Prison.

| name of project |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MWP STAFF INITIALS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITEMS |  |  |  | NUMBER OF PIECES |  | in | out | in | out | in | out | in | out | in | out | in | out | in | out |

Attachment B